STANDARD OPERATING PROCEDURE (SOP)

Division: Centre for Construction and Engineering Technologies (CCET)
School:  Apprenticeship and Skilled Trades (AST)

Last updated: 	April 24, 2020

ATTENTION: 	DO NOT use this equipment unless your Professor OR an approved Technologist has provided

training on the Standard Operating Procedure (SOP) including a practical demonstration.

| MACHINE/EQUIPMENT: |  |  |  |  |  | Dewalt 4.5 inch Angle Grinder |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROOM/LAB: |  |  |  |  |  | D110 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | POTENTIAL HAZARDS: - Avoid using gloves - No loose clothing or jewellery - Long hair must be tied back - Avoid accidental starting. Be sure switch is off before plugging in. - Sparks, metal and disc material projectiles - “Kickback” or biting/binding of disc or stone - Avoid bouncing the wheel or giving it rough treatment |  |  |  |  |  |  |
| PERSONAL PROTECTIVE EQUIPMENT (PPE) – CHECK ALL REQUIRED |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety Footwear | Protective Clothing | Eye Protection |  | Face Shield |  |  |  | Ear protection | Dust Mask |  | Gloves |  | Welding Mask |
| MANUFACTUER MANUAL: |  | Name of manual |  |  |  |  |  |  |  |  |  |  |  |
| ADDITIONAL RESOURCES: |  | “Use of Portable Grinders” www.ccohs.ca. “Abrasive Grinding Wheels” www.wsps.ca |  |  |  |  |  |  |  |  |  |  |  |
| STANDARD OPERATING PROCEDURE (STUDENTS & EMPLOYEES) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRE-OPERATIONAL: - Always use proper guard with grinding wheel. Always use side handle. Tighten the handle securely - Use clamps or other practical way to secure and support the workpiece to a stable platform - Check for misalignment or binding of moving parts, breakage of parts, and any other condition that may affect the tool’s operation. - Check cord for splices and exposed wires OPERATIONAL: - Make sure both paddles are engaged and pressed down. Grinder will not start otherwise - Maintain a firm grip with both hands on the unit and position your body and arm to allow you to resist kickback forces - Allow the tool to reach full speed before touching the tool to the work surface - Maintain a 20˚ to 30˚ angle between the tool and work surface with minimum pressure to the work surface, allowing the tool to operate at high speed - Continuously move the tool in a forward and back motion to avoid creating gouges in the work surface (grinding stone) - Direct sparks away from operator, bystanders or flammable materials SHUT-DOWN: - Remove the tool from work surface before turning tool off. Allow the tool to stop rotating before laying it down - Clean out your tool often, especially after heavy use |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STANDARD OPERATING PROCEDURE (EMPLOYEES ONLY) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PREVENTATIVE MAINTENANCE: - Blowing dust and grit out of motor and switch actuator using clean, dry compressed air is a necessary regular maintenance procedure - Never use solvents or other harsh chemicals for cleaning the non-metallic parts of the tool. Use a clean, dry cloth only. - Regular checks on all moving parts of grinder and cables is recommended |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMPLOYEES AUTHORIZED TO TRAIN AND OVERSEE OPERATORS ON MACHINE/EQUIPMENT SOP |  |  |  |  |  |  |  |  |  |  |  |  |  |
| JOB TITLE |  |  | NAME |  |  |  |  |  |  | GBC PHONE OR EMAIL |  |  |  |
| APPROVAL - DEPARTMENT CHAIR OR DEAN |  |  |  |  | NAME |  |  |  |  |  |  | DATE |  |
|  |  |  |  |  |  |  |  |  |  |  |  | TBD |  |

APPROVED STANDARD OPERATING PROCEDURE (SOP) MUST BE POSTED WITH MACHINE/EQUIPMENT